# LiDAR–Camera Sensor Fusion

Research Paper Section — AVMI UGV Dataset

To provide complementary geometric and semantic understanding of the environment, we implement a LiDAR–camera fusion pipeline that projects 3-D point cloud data onto the 2-D camera image plane. The fused representation assigns each projected LiDAR point its semantic label — encoded directly in the point cloud — enabling validation of the terrain segmentation model and geometric grounding of pixel-wise predictions.

## Sensor Configuration

The simulation platform (Unreal Engine) equips the UGV with a semantic LiDAR and an RGB camera mounted at fixed offsets on the vehicle body. In the UE vehicle frame (X-forward, Y-right, Z-up), the LiDAR is located at (−31.08, 0.00, 211.00) cm and the camera at (55.00, −15.00, 200.00) cm, with a camera pitch of −2°. The sensor separation of approximately 86 cm along the vehicle longitudinal axis defines the system baseline. The camera intrinsic matrix is:

| 278.59    0     323.80 |

K =  |   0     334.29  229.20 |

|   0       0       1    |

where fx = 278.59 and fy = 334.29 are the focal lengths in pixels, and (cx, cy) = (323.80, 229.20) is the principal point, all derived from the Unreal Engine camera calibration.

## Coordinate Transformation

To project LiDAR points into the camera frame, we derive the extrinsic rotation R(L→C) and translation t(L→C) from the known sensor placements. UE world coordinates (cm, Y-right) are first converted to the ROS vehicle frame (metres, Y-left):

p_ROS = [ x/100,  -y/100,  z/100 ]ᵀ

The fixed rotation from the ROS vehicle frame (X-forward, Y-left, Z-up) to the camera optical frame (X-right, Y-down, Z-forward) is the matrix:

|  0  -1   0 |

R_base  =  |  0   0  -1 |

|  1   0   0 |

The camera's in-vehicle rotation R_cam (pitch −2°) is computed from ZYX Euler angles. The full LiDAR-to-camera extrinsics are then:

R(L→C) = R_base · R_cam

t(L→C) = R(L→C) · (p_C − p_L)

where p_C and p_L are the camera and LiDAR positions converted to the ROS vehicle frame.

## Projection and Filtering

Each LiDAR point P_L = [x, y, z]ᵀ is transformed into the camera frame as:

P_C = R(L→C) · P_L  +  t(L→C)

Points with depth Z_C < 0.5 m (behind or too close to the camera) and Z_C > 50 m are discarded. The remaining points are projected onto the image plane via the standard pinhole camera model:

u = fx · (X_C / Z_C) + cx

v = fy · (Y_C / Z_C) + cy

Only points whose pixel coordinates (u, v) fall within the image bounds are retained for rendering and publication.

## Semantic Point Cloud Coloring

The Unreal Engine semantic LiDAR encodes a terrain class label for each point as a packed RGB value stored in the fourth channel of the point cloud (IEEE 754 float32, interpreted as 0xRRGGBB). Three terrain classes are represented, consistent with the six-class taxonomy used for segmentation training:

Rock: Red   (R=255, G=0, B=0)

Ground / Traversable terrain: Brown  (R=139, G=90, B=43)

Vegetation / Trees: Green  (R=0, G=128, B=0)

The RGB channels are extracted from the packed 32-bit integer via bitwise operations:

r = (rgb >> 16) & 0xFF

g = (rgb >>  8) & 0xFF

b = (rgb      ) & 0xFF

Each surviving projected point is rendered on the camera image at (u, v) using its semantic colour, producing a fused overlay that directly visualises the geometric extent and class identity of each terrain region (Fig. X).

## Limitations: Parallax-Induced Boundary Displacement

Due to the physical separation between the LiDAR and camera origins, a parallax-induced boundary displacement arises at object edges. The pixel displacement is approximated by:

Δu ≈ (fx · b) / Z

where b is the lateral sensor baseline and Z is the object depth. For the AVMI sensor configuration with a lateral offset of 15 cm, the expected displacement at Z = 5 m is approximately 8 pixels, decreasing inversely with distance. This effect is inherent to multi-sensor systems with non-coincident mounting positions and is a well-documented characteristic of LiDAR–camera fusion [1]. Interior points within object regions are correctly labelled; only boundary pixels are affected.

## References

[1] A. Geiger, P. Lenz, and R. Urtasun, "Are we ready for autonomous driving? The KITTI vision benchmark suite," in Proc. IEEE CVPR, 2012.