# AVMI UGV Dataset: Terrain Taxonomy, Annotation, and Cross-Dataset Evaluation

[ Draft section for insertion into main paper ]

# Section III — AVMI UGV Dataset and Terrain Taxonomy

## A.  Terrain Class Ontology

Existing off-road datasets define broad ontologies intended for general environment description. RUGD [1] provides 24 terrain and object classes while RELLIS-3D [2] provides 20 classes, yet both suffer from severe pixel-level class imbalance — dominant classes such as grass or sky can occupy over 80% of all labeled pixels, while navigation-relevant classes like rock or stump contribute less than 1%. This imbalance makes training difficult and leads to models that specialise on high-frequency classes while ignoring rare but important ones.

To address this, the AVMI UGV dataset adopts a compact six-class visual taxonomy designed specifically around what a wheeled ground vehicle needs to perceive for safe outdoor navigation. The classes are chosen to be visually distinct, navigation-relevant, and achievable with a balanced pixel distribution in simulation:

Sky —  Open sky and clouds. Always non-traversable; provides spatial context.

Tree —  Tree trunks and canopy. Non-traversable; key landmark in outdoor environments.

Bush —  Low shrubs and dense ground-level vegetation. Treated conservatively as an obstacle.

Ground —  Open grass and bare earth. The primary traversable class for the UGV.

Rock —  Exposed rock faces and loose rock beds. Traversable only with caution.

Stump —  Wooden stumps, logs, and similar solid low-lying objects. Non-traversable.

Table 1 — Class ontology: AVMI vs RUGD vs RELLIS-3D

| AVMI Class | RUGD equivalent classes (of 24) | RELLIS-3D equivalent classes (of 20) |
| --- | --- | --- |
| Sky | sky | sky |
| Tree | tree | tree |
| Bush | bush | bush |
| Ground | dirt, grass, gravel (see §III-C) | dirt, mud |
| Rock | rock, rock-bed | rubble |
| Stump | log | log |
| — | remaining 18 classes (asphalt, concrete, vehicle, fence…) | remaining 14 classes (grass, asphalt, pole, vehicle, fence…) |

## B.  Dataset Collection and Annotation

The AVMI UGV dataset is collected using Unreal Engine (UE) simulation rendered from the first-person perspective of a ground vehicle traversing open grassy terrain with trees, rocks, and stumps. Simulation allows pixel-perfect annotations without manual labelling effort and gives direct control over lighting, vegetation density, and class balance — an advantage over purely real-world datasets where rare classes may be present in very few frames.

Each frame is annotated at full pixel precision using UE's semantic segmentation pass, which assigns a unique RGB colour per class. At training time, annotations are converted to integer class indices via nearest-colour L2 matching in RGB space to handle minor JPEG compression artefacts. Frames with severe motion blur or fewer than two visible classes are excluded. The retained frames are split into training, validation, and held-out test sets.

## C.  Cross-Dataset Class Mapping

To evaluate how well the AVMI-trained model generalises to real outdoor data, we fine-tune it on RUGD and RELLIS-3D by remapping their annotations to our six-class scheme. Because both datasets use different and broader ontologies, each source class is assigned to the closest AVMI class or marked as ignore (255). The complete class-to-class mapping for both datasets is given in Table 2.

Table 2 — Class mapping applied to RUGD and RELLIS-3D annotation files

| AVMI Target | RUGD source classes | RELLIS-3D source classes (sequential index) |
| --- | --- | --- |
| Sky (0) | sky (7) | sky (seq. 6) |
| Tree (1) | tree (4) | tree (seq. 3) |
| Bush (2) | bush (19) | bush (seq. 14) |
| Ground (3) | dirt (1), grass (3), gravel (11), asphalt (10), concrete (23), mulch (13) | dirt (seq. 1), mud (seq. 18) |
| Rock (5) | rock (21), rock-bed (14), water (6) | rubble (seq. 19) |
| Stump (4) | log (15), pole (5), vehicle (8), fence (18), building (12), sign (20) | log (seq. 11) |
| Ignore (255) | sand (2), container (9), bicycle (16), person (17), bridge (22), picnic-table (24) | grass (seq. 2), asphalt (seq. 9), concrete (seq. 15), pole (seq. 4), water (seq. 5), vehicle (seq. 7), barrier (seq. 16), puddle (seq. 17), person (seq. 12), fence (seq. 13) |
| Note | RUGD _orig.png stores direct class indices (0–24) | RELLIS _orig.png stores sequential indices 0–19, NOT the original paper IDs (up to 34) |

Important: RELLIS-3D annotation files (_orig.png) use sequential indices 0–19, not the original non-sequential paper class IDs (which go up to 34 with gaps). An initial implementation used the paper IDs directly, resulting in completely incorrect label assignments (e.g., pole pixels trained as tree, vehicle as sky). Correcting to sequential indices was essential for training to converge.

# Section IV — Model Architecture and Training

## A.  Network Architecture

We adopt GANav [3], an attention-based segmentation network designed for texture-driven terrain boundaries. The backbone is MixVisionTransformer-B0 (MiT-B0) with four hierarchical feature stages (embedding dimension 32, spatial reduction ratios [8,4,2,1]). The decode head (OursHeadClassAtt) fuses multi-scale features and applies bi-directional Polarised Self-Attention (PSA) over a fixed 97×97 attention mask with 384 channels. Two lightweight FCN auxiliary heads at intermediate backbone stages provide deep supervision (loss weight 0.4 each). Input images are cropped to 300×375 and normalised with ImageNet statistics. Total: ~3.7M parameters.

## B.  Training from Scratch on AVMI Dataset

The model is trained from scratch on the AVMI UGV dataset for 240,000 iterations with no external pretrained weights. We use SGD (lr=0.06, momentum=0.9, weight decay=4×10⁻⁵, batch size=4) with polynomial LR decay and linear warmup over 1,500 iterations. This establishes an AVMI-specific feature representation that serves as the initialisation for all cross-dataset fine-tuning experiments.

Table 3 — AVMI base model: per-class validation IoU (%) at 240,000 iterations

|  | Sky | Tree | Bush | Ground | Rock | Stump | mIoU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IoU (%) | 86.9 | 70.2 | 4.9 | 91.2 | 40.4 | 15.5 | 51.51 |

Sky and Ground are well-learned (high frequency, visually distinct). Tree reaches 70.2% IoU from simulation rendering consistency. Bush and Stump remain difficult due to diffuse boundaries and low pixel count.

## C.  Cross-Dataset Fine-Tuning

Starting from the AVMI base model, we fine-tune separately on RUGD and RELLIS-3D using the class mapping described in Section III-C. All fine-tuning runs use SGD (lr=0.003, polynomial decay, batch size=4) for 100,000 iterations. The crop size is kept at 300×375 to match the PSA attention mask from AVMI training.

We evaluated two strategies for each dataset: (1) mapping all source classes to the nearest AVMI class, and (2) mapping only visually consistent classes while ignoring the rest. Table 4 summarises the validation mIoU across all experiments.

Table 4 — Cross-dataset fine-tuning results: validation IoU (%) after 100k iterations

| Model | Strategy | Sky | Tree | Bush | Ground | Rock | Stump | mIoU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AVMI Base | Scratch (240k) | 86.9 | 70.2 | 4.9 | 91.2 | 40.4 | 15.5 | 51.51 |
| RUGD → AVMI | Full Mapping | 57.0 | 5.6 | 0.0 | 88.4 | 74.3 | 68.7 | 49.0 |
| RUGD → AVMI | Selective Mapping | 87.4 | 58.7 | 5.2 | 91.6 | 93.6 | n/a | 67.4 |
| RUGD → AVMI | Full + Class Weights | 49.2 | 10.4 | 6.7 | 84.9 | 69.2 | 69.9 | 48.4 |
| RELLIS → AVMI | Full Mapping (wrong index) | — | — | — | — | — | — | — |
| RELLIS → AVMI | Fixed Sequential Index | 71.2 | 61.1 | 20.8 | 63.8 | 0.0 | 0.5 | 36.2 |

Yellow row = best result for cross-domain generalisation to UGV images. — indicates the experiment failed to converge due to incorrect annotation indexing.

The RUGD fine-tuning experiments struggled due to severe class imbalance in the source dataset. In RELLIS-3D, an indexing inconsistency between the dataset paper (non-sequential IDs up to 34) and the stored annotation files (sequential IDs 0–19) caused the initial mapping to train on entirely wrong labels. After correcting the index mapping, the RELLIS-3D fine-tuned model achieved sky IoU of 71.2%, tree IoU of 61.1%, and ground IoU of 63.8% on the RELLIS-3D validation set, and produced visually coherent terrain segmentation on held-out UGV images (sky 18–29%, tree 15–30%, ground 47–58%).

[Figure X — Insert comparison grid here: original image | AVMI scratch | RELLIS fixed]

Fig. X — Segmentation output on held-out UGV images. Left: original. Middle: AVMI scratch model. Right: RELLIS fixed-index fine-tuned model. The fine-tuned model correctly separates sky, tree canopy, and traversable ground in environments not seen during training.